1. Lab Duration

Hours Spent: 8 Hours

2. Processor Hierarchy & Block Diagram

The processor is built using a modular approach to separate the control logic from the data processing units.

- top: Connects the RISC-V core to the unified memory .

- riscv: The core CPU containing the controller and datapath.

- controller: Manages the FSM states and decodes ALU instructions.

- datapath: Contains the ALU, Register File, and instruction/data registers.

Processor Block Diagram

3. Hierarchical SystemVerilog Description

The following modules were implemented to create the multicycle architecture.

- top.sv:

module top (

input  logic        clk, reset,

output logic [31:0] WriteData, DataAdr,

output logic        MemWrite

);

logic [31:0] ReadData;

// Instantiate the RISC-V multicycle processor core

riscv rv (clk, reset, DataAdr, WriteData, MemWrite, ReadData);

// Instantiate the unified memory (Instruction + Data)

mem m (clk, MemWrite, DataAdr, WriteData, ReadData);

endmodule

- mem.sv:

module mem (

input  logic        clk, we,

input  logic [31:0] a, wd,

output logic [31:0] rd

);

logic [31:0] RAM[63:0];

// Load the machine code file

initial

$readmemh("riscvtest.txt", RAM);

// Read logic: Word aligned (ignores bottom two bits)

assign rd = RAM[a[31:2]];

// Synchronous write logic

always_ff @(posedge clk)

if (we) RAM[a[31:2]] <= wd;

endmodule

- riscv.sv:

module riscv (

input  logic        clk, reset,

output logic [31:0] Adr, WriteData,

output logic        MemWrite,

input  logic [31:0] ReadData

);

// Internal Control Wires between Controller and Datapath

logic [1:0]  ResultSrc, ALUSrcA, ALUSrcB;

logic [2:0]  ALUControl, ImmSrc;

logic        RegWrite, PCWrite, AdrSrc, IRWrite;

logic        Zero;

logic [31:0] Instr;

// Instantiate the Controller (FSM)

controller c (

clk, reset,

Instr[6:0], Instr[14:12], Instr[30], // op, funct3, funct7

Zero,

ImmSrc, ALUSrcA, ALUSrcB, ResultSrc,

AdrSrc, ALUControl, IRWrite, PCWrite, RegWrite, MemWrite

);

// Instantiate the Datapath (Hardware blocks)

datapath dp (

clk, reset,

ImmSrc, ALUSrcA, ALUSrcB, ResultSrc,

AdrSrc, ALUControl, IRWrite, PCWrite, RegWrite,

Zero, Instr, Adr, WriteData, ReadData

);

endmodule

- controller.sv:

module controller(

input  logic       clk, reset,

input  logic [6:0] op,

input  logic [2:0] funct3,

input  logic       funct7b5,

input  logic       Zero,

output logic [2:0] ImmSrc,

output logic [1:0] ALUSrcA, ALUSrcB,

output logic [1:0] ResultSrc,

output logic       AdrSrc,

output logic [2:0] ALUControl,

output logic       IRWrite, PCWrite, RegWrite, MemWrite

);

logic [1:0] ALUOp;

logic       Branch, PCUpdate;

// Main Decoder FSM (Using NAMED mapping for safety)

mainfsm fsm(

.clk(clk), .reset(reset), .op(op),

.ImmSrc(ImmSrc), .ALUSrcA(ALUSrcA), .ALUSrcB(ALUSrcB), .ResultSrc(ResultSrc),

.AdrSrc(AdrSrc), .IRWrite(IRWrite), .PCUpdate(PCUpdate),

.RegWrite(RegWrite), .MemWrite(MemWrite), .Branch(Branch), .ALUOp(ALUOp)

);

// ALU Decoder

aludec ad(op[5], funct3, funct7b5, ALUOp, ALUControl);

// Branch logic for PCUpdate

assign PCWrite = (Branch & Zero) | PCUpdate;

endmodule

- mainfsm.sv:

module mainfsm(

input  logic       clk, reset,

input  logic [6:0] op,

output logic [2:0] ImmSrc,

output logic [1:0] ALUSrcA, ALUSrcB, ResultSrc,

output logic       AdrSrc, IRWrite, PCUpdate, RegWrite, MemWrite, Branch,

output logic [1:0] ALUOp

);

// State definitions based on Table 1 and RISC-V multicycle flow

typedef enum logic [3:0] {

S0_FETCH, S1_DECODE, S2_MEMADR, S3_MEMREAD, S4_MEMWB, S5_MEMWRITE,

S6_EXECUTER, S7_ALUWB, S8_EXECUTEI, S9_JAL, S10_BEQ

} statetype;

statetype state, nextstate;

// --- State Register ---

always_ff @(posedge clk or posedge reset)

if (reset) state <= S0_FETCH;

else       state <= nextstate;

// --- Next State Logic ---

always_comb

case (state)

S0_FETCH:   nextstate = S1_DECODE;

S1_DECODE:  case(op)

7'b0000011: nextstate = S2_MEMADR;   // lw

7'b0100011: nextstate = S2_MEMADR;   // sw

7'b0110011: nextstate = S6_EXECUTER; // R-type

7'b0010011: nextstate = S8_EXECUTEI; // I-type (addi)

7'b1101111: nextstate = S9_JAL;      // jal

7'b1100011: nextstate = S10_BEQ;     // beq

default:    nextstate = S0_FETCH;

endcase

S2_MEMADR:  if (op == 7'b0000011) nextstate = S3_MEMREAD;

else                  nextstate = S5_MEMWRITE;

S3_MEMREAD:  nextstate = S4_MEMWB;

S4_MEMWB:    nextstate = S0_FETCH;

S5_MEMWRITE: nextstate = S0_FETCH;

S6_EXECUTER: nextstate = S7_ALUWB;

S7_ALUWB:    nextstate = S0_FETCH;

S8_EXECUTEI: nextstate = S7_ALUWB;

S9_JAL:      nextstate = S7_ALUWB;

S10_BEQ:     nextstate = S0_FETCH;

default:     nextstate = S0_FETCH;

endcase

// --- Output Logic (Control Signals) ---

// Signals mapped according to Figure 1

always_comb begin

// Default values to ensure no latches are created

{AdrSrc, IRWrite, RegWrite, MemWrite, PCUpdate, Branch} = 6'b0;

{ALUSrcA, ALUSrcB, ResultSrc, ALUOp} = 8'b0;

case (state)

S0_FETCH: begin

AdrSrc = 1'b0;     // Select PC as address

IRWrite = 1'b1;    // Enable Instruction Register write

ALUSrcA = 2'b00;   // SrcA = PC

ALUSrcB = 2'b10;   // SrcB = 4

ALUOp = 2'b00;     // Add

ResultSrc = 2'b10; // Result = ALUResult

PCUpdate = 1'b1;   // Trigger PC update

end

S1_DECODE: begin

ALUSrcA = 2'b01;   // SrcA = OldPC

ALUSrcB = 2'b01;   // SrcB = ImmExt

ALUOp = 2'b00;     // Add (pre-calculate branch target)

End

S2_MEMADR: begin

ALUSrcA = 2'b10;   // SrcA = Reg RD1

ALUSrcB = 2'b01;   // SrcB = ImmExt

ALUOp = 2'b00;     // Add (calculate memory address)

end

S3_MEMREAD: begin

AdrSrc = 1'b1;     // Select ALUOut as address

ResultSrc = 2'b00;

end

S4_MEMWB: begin

ResultSrc = 2'b01; // Result = Data from memory

RegWrite = 1'b1;   // Write to register file

end

S5_MEMWRITE: begin

AdrSrc = 1'b1;     // Select ALUOut as address

MemWrite = 1'b1;   // Enable memory write

ResultSrc = 2'b00;

end

S6_EXECUTER: begin

ALUSrcA = 2'b10;   // SrcA = Reg RD1

ALUSrcB = 2'b00;   // SrcB = Reg RD2

ALUOp = 2'b10;     // Logic defined by funct3/7

end

S7_ALUWB: begin

ResultSrc = 2'b00; // Result = ALUOut

RegWrite = 1'b1;   // Write to register file

end

S8_EXECUTEI: begin

ALUSrcA = 2'b10;   // SrcA = Reg RD1

ALUSrcB = 2'b01;   // SrcB = ImmExt

ALUOp = 2'b10;     // Logic defined by funct3

end

S9_JAL: begin

ALUSrcA = 2'b01;   // SrcA = OldPC

ALUSrcB = 2'b10;   // SrcB = 4

ALUOp = 2'b00;     // Add

ResultSrc = 2'b00; // Result = ALUOut

PCUpdate = 1'b1;   // Jump update

end

S10_BEQ: begin

ALUSrcA = 2'b10;   // SrcA = Reg RD1

ALUSrcB = 2'b00;   // SrcB = Reg RD2

ALUOp = 2'b01;     // Subtract (for Zero comparison)

ResultSrc = 2'b00;

Branch = 1'b1;     // Enable branch logic

end

endcase

end

// --- Immediate Decoder ---

// Logic for ImmSrc based on instruction type

always_comb

case(op)

7'b0000011: ImmSrc = 3'b000; // I-type (lw)

7'b0010011: ImmSrc = 3'b000; // I-type (addi)

7'b0100011: ImmSrc = 3'b001; // S-type (sw)

7'b0110011: ImmSrc = 3'bxxx; // R-type

7'b1100011: ImmSrc = 3'b010; // B-type (beq)

7'b1101111: ImmSrc = 3'b011; // J-type (jal)

default:    ImmSrc = 3'b000;

endcase

endmodule

- aludec.sv:

module aludec(

input  logic       opb5,

input  logic [2:0] funct3,

input  logic       funct7b5,

input  logic [1:0] ALUOp,

output logic [2:0] ALUControl

);

logic  RtypeSub;

assign RtypeSub = funct7b5 & opb5; // TRUE for SUB instruction

always_comb

case(ALUOp)

2'b00: ALUControl = 3'b000; // addition

2'b01: ALUControl = 3'b001; // subtraction

default: case(funct3) // R-type or I-type ALU

3'b000: if (RtypeSub) ALUControl = 3'b001; // sub

else          ALUControl = 3'b000; // add

3'b010: ALUControl = 3'b101; // slt

3'b110: ALUControl = 3'b011; // or

3'b111: ALUControl = 3'b010; // and

default: ALUControl = 3'bxxx; // unknown

endcase

endcase

endmodule

- datapath.sv:

module datapath (

input  logic        clk, reset,

input  logic [2:0]  ImmSrc,

input  logic [1:0]  ALUSrcA, ALUSrcB,

input  logic [1:0]  ResultSrc,

input  logic        AdrSrc,

input  logic [2:0]  ALUControl,

input  logic        IRWrite, PCWrite, RegWrite,

output logic        Zero,

output logic [31:0] Instr,

output logic [31:0] Adr, WriteData,

input  logic [31:0] ReadData

);

logic [31:0] PC, OldPC;

logic [31:0] Data, RD1, RD2, A, SrcA, SrcB;

logic [31:0] ALUResult, ALUOut, Result, ImmExt;

// --- Memory and Instruction Logic ---

// PC Register: Updates only when PCWrite is high

flopenr #(32) pcreg(clk, reset, PCWrite, Result, PC);

// Address Mux: Selects between PC (Fetch) and Result (Mem Access)

mux2 #(32)   adrmux(PC, Result, AdrSrc, Adr);

// Instruction Register: Captures new instruction from memory

flopenr #(32) ir(clk, reset, IRWrite, ReadData, Instr);

flopenr #(32) oldpcreg(clk, reset, IRWrite, PC, OldPC);

// Data Register: Buffer for data read from memory

flopr #(32)  datareg(clk, reset, ReadData, Data);

// --- Register File and Extend Logic ---

regfile      rf(clk, RegWrite, Instr[19:15], Instr[24:20],

Instr[11:7], Result, RD1, RD2);

extend       ext(Instr[31:7], ImmSrc, ImmExt);

// Pipelines registers to hold values for the Execute stage

flopr #(32)  areg(clk, reset, RD1, A);

flopr #(32)  wdreg(clk, reset, RD2, WriteData);

// --- ALU Logic ---

// ALUSrcA Mux: Selects PC, OldPC, or Register A

mux3 #(32)   srcamux(PC, OldPC, A, ALUSrcA, SrcA);

// ALUSrcB Mux: Selects Register B (WriteData), ImmExt, or constant 4

mux3 #(32)   srcbmux(WriteData, ImmExt, 32'd4, ALUSrcB, SrcB);

alu          alu_inst(SrcA, SrcB, ALUControl, ALUResult, Zero);

// ALUOut register: Stores ALU result for use in a later cycle

flopr #(32)  aluoutreg(clk, reset, ALUResult, ALUOut);

// --- Result Logic ---

// ResultSrc Mux: Final value to be written to PC or RegFile

mux3 #(32)   resmux(ALUOut, Data, ALUResult, ResultSrc, Result);

endmodule

- buildingblocks.sv:

// 2-way Multiplexer

module mux2 #(parameter WIDTH = 8) (

input  logic [WIDTH-1:0] d0, d1,

input  logic             s,

output logic [WIDTH-1:0] y

);

assign y = s ? d1 : d0;

endmodule

// 3-way Multiplexer

module mux3 #(parameter WIDTH = 8) (

input  logic [WIDTH-1:0] d0, d1, d2,

input  logic [1:0]       s,

output logic [WIDTH-1:0] y

);

assign y = (s == 2'b00) ? d0 :

(s == 2'b01) ? d1 : d2;

endmodule

// Resettable Flip-Flop

module flopr #(parameter WIDTH = 8) (

input  logic             clk, reset,

input  logic [WIDTH-1:0] d,

output logic [WIDTH-1:0] q

);

always_ff @(posedge clk, posedge reset)

if (reset) q <= 0;

else       q <= d;

endmodule

// Resettable Flip-Flop with Enable

module flopenr #(parameter WIDTH = 8) (

input  logic             clk, reset, en,

input  logic [WIDTH-1:0] d,

output logic [WIDTH-1:0] q

);

always_ff @(posedge clk, posedge reset)

if (reset)   q <= 0;

else if (en) q <= d;

endmodule

- regfile.sv:

module regfile(

input  logic        clk,

input  logic        we3,

input  logic [4:0]  a1, a2, a3,

input  logic [31:0] wd3,

output logic [31:0] rd1, rd2

);

logic [31:0] rf[31:0];

// Three-ported register file

// Read two ports combinationally (A1/RD1, A2/RD2)

// Write third port on rising edge of clock (A3/WD3/WE3)

// Register 0 hardwired to 0

always_ff @(posedge clk)

if (we3) rf[a3] <= wd3;

assign rd1 = (a1 != 0) ? rf[a1] : 0;

assign rd2 = (a2 != 0) ? rf[a2] : 0;

endmodule

- alu.sv:

module alu(

input  logic [31:0] a, b,

input  logic [2:0]  alucontrol,

output logic [31:0] result,

output logic        zero

);

logic [31:0] condinvb, sum;

logic        v;              // overflow

logic        isAddSub;       // true of addition or subtraction

assign condinvb = alucontrol[0] ? ~b : b;

assign sum = a + condinvb + alucontrol[0];

assign isAddSub = ~alucontrol[2] & ~alucontrol[1] |

~alucontrol[2] & alucontrol[0];

always_comb

case (alucontrol)

3'b000:  result = sum;               // add

3'b001:  result = sum;               // subtract

3'b010:  result = a & b;             // and

3'b011:  result = a | b;             // or

3'b101:  result = sum[31];           // slt

default: result = 32'bx;

endcase

assign zero = (result == 32'b0);

endmodule

- extend.sv:

module extend(

input  logic [31:7] instr,

input  logic [2:0]  immsrc,

output logic [31:0] immext

);

always_comb

case(immsrc)

3'b000:  immext = {{20{instr[31]}}, instr[31:20]}; // I-type (addi, lw)

3'b001:  immext = {{20{instr[31]}}, instr[31:25], instr[11:7]}; // S-type (sw)

3'b010:  immext = {{19{instr[31]}}, instr[31], instr[7], instr[30:25], instr[11:8], 1'b0}; // B-type (beq)

3'b011:  immext = {{11{instr[31]}}, instr[31], instr[19:12], instr[20], instr[30:21], 1'b0}; // J-type (jal)

default: immext = 32'bx;

endcase

endmodule

4. Table 1: Expected Operation

This table tracks the first three instructions of the test program (addi operations).

| Step | PC | Instr | State | Result | Result Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | 00 | 00500113 | S0: Fetch | 4 | ALU calculates PC + 4 |
| 2 | 04 | 00500113 | S1: Decode | X | ALU calculates OldPC + Imm |
| 3 | 04 | 00500113 | S8: ExecuteI | 5 | ALUResult = 0 + 5 |
| 4 | 04 | 00500113 | S7: ALUWB | 5 | Register x2 written with 5 |
| 5 | 04 | 00C00193 | S0: Fetch | 8 | ALU calculates PC + 4 |
| 6 | 08 | 00C00193 | S1: Decode | X | ALU calculates OldPC + Imm |
| 7 | 08 | 00C00193 | S8: ExecuteI | 12 | ALUResult = 0 + 12 |
| 8 | 08 | 00C00193 | S7: ALUWB | 12 | Register x3 written with 12 |
| 9 | 08 | FF718393 | S0: Fetch | 12 | ALU calculates PC + 4 |
| 10 | 0C | FF718393 | S1: Decode | X | ALU calculates OldPC + Imm |
| 11 | 0C | FF718393 | S8: ExecuteI | 3 | ALUResult = 12 - 9 |
| 12 | 0C | FF718393 | S7: ALUWB | 3 | Register x7 written with 3 |

5. Simulation Waveforms

The simulation was performed in Questa to verify the timing and functional accuracy of the control signals .

Full Simulation Overview:

Partial Simulation Overviews:

- 0ns - 150ns:

Simulation result in 0ns - 150ns

- 140ns - 290ns

Simulation result in 140ns - 290ns

- 280ns - 430ns:

Simulation result in 280ns - 430ns

- 420ns - 570ns:

Simulation result in 420ns - 570ns

- 560ns - 710ns:

Simulation result in 560ns - 710ns

- 700ns - 725ns:

WriteData shows 0x19 in the end it means 25 in decimal

DataAdr shows 0x64 in the end it means 100 in decimal.

Simulation result in 700ns - 725ns

Transcript Console Output

- Description: The Questa transcript confirms the successful completion of the test program. The simulation reached the $stop command at line 30 of riscv_testbench.sv exactly at 725 ns, printing the message "Simulation succeeded". This indicates that the processor successfully navigated all R-type, I-type, and Branch instructions to reach the final memory write.

6. Conclusion & Debugging

The multicycle RISC-V processor successfully passed the verification suite .

- Logic Verification: The testbench reported "Simulation succeeded" at 725 ns, confirming that all instructions (R-type, I-type, Load/Store, and Branch) are functioning as intended .

- Debugging: A critical race condition was found in the OldPC logic. By converting the OldPC register to a flopenr and using IRWrite as an enable, the processor now correctly maintains the current instruction's address for jal return calculations.

- Final Result: The final memory write to address 100 with value 25 proves the overall architectural integrity of the design .

Git Repository Link

URL: https://github.com/Alperen-Salman/ELE432_Lab3_prework